Wearable-Free Fall Detection System

User Guide

ESP32-based sensor fusion for privacy-preserving fall detection

Contents

1. Goals	3

2. Overview of Design	3

Hardware Overview	3

Software Overview	3

3. Components Required	4

4. Hardware Setup	4

Connection Diagram	5

DEVKIT V1 Pinout	5

Example Breadboard Implementation	5

Example Final Implementation	6

5. Software Setup	6

Dependencies	6

Building & Flashing	6

Operating Modes	7

6. Tuning	7

Microphone Tuning	7

CSI Tuning	8

Vibration Tuning	9

7. Final Product	11

Operating the Device	11

8. Resources	12

GitHub Repository	12

Component Purchases	12

Espressif / ESP-IDF Documentation	12

CSI Sensing Background	12

# 1. Goals

This project creates an inexpensive, camera-free fall detection system that does not require wearables. Because the elderly often forget or choose not to wear health devices, a system installable in a home or assisted living facility can be more reliable. By avoiding cameras, the design also preserves resident privacy. Additionally, by using simple Wi-Fi hardware rather than expensive radar hardware, the system can be made inexpensively.

# 2. Overview of Design

## Hardware Overview

This system uses three sensing modalities to detect a fall:

- Piezoelectric vibration sensor: detects impact with the floor

- Microphone: detects the thump of a fall or presence in the room

- Wi-Fi CSI (Channel State Information): detects motion-induced changes in signal propagation

An ESP32 serves as the central processing hub, and a speaker plays an audible alert on fall detection.

💡 For more background on CSI sensing technology, see the Resources section.

## Software Overview

ESP32 Firmware (C) (folder in repo: code/node_final)

Handles all on-device processing:

- Interfacing with the microphone and speaker

- Reading and processing vibration data

- Sending and receiving UDP packets

- Processing CSI data

- Development modes: streaming raw sensor data for tuning

- Normal operation mode: transmitting only detection results to preserve privacy

Python Scripts (folder in repo: code/data_proc)

Run on a computer connected to the ESP32 via USB:

- Dashboard showing sensor activity and detection status (normal mode)

- Visualization tools for CSI, microphone, and vibration data (development modes)

# 3. Components Required

The following components are needed. Purchase links are in the Resources section.

| Component | Details |
| --- | --- |
| ESP32 DEVKIT V1 | ×2 Main microcontroller boards |
| INMP441 Board | I2S MEMS Microphone |
| Piezoelectric Ceramic Disc | Vibration / impact sensor |
| MAX98357 Amplifier Board | I2S audio amplifier |
| Speaker | 1 W, 8 Ohm |

# 4. Hardware Setup

Connect components as shown in the diagrams below. A breadboard is recommended for initial testing before creating a permanent implementation. Optional 3D-printable enclosure CAD files are available in the project repository.

💡 Only the receiver ESP32 needs sensors connected. The second ESP32 acts solely as a Wi-Fi packet transmitter.

## Connection Diagram

## DEVKIT V1 Pinout

## Example Breadboard Implementation

## Example Final Implementation

# 5. Software Setup

## Dependencies

- Install ESP-IDF using the official Espressif getting started guide (link in Resources). If the installer fails, clone the esp-idf repository manually and run:

install.ps1

- Clone the thread_proj repository from the project GitHub (link in Resources).

## Building & Flashing

💡 If using the ESP-IDF GUI rather than CLI, refer to Espressif’s IDE documentation.

Run the following commands from the code/node_final directory. The export script must be run once per terminal session (add esp-idf directory to PATH for convenience):

export.ps1                              # Run once per terminal session

idf.py set-target esp32

idf.py menuconfig                       # Component Config > Wi-Fi > Enable CSI

idf.py build

idf.py flash

💡 Flash this firmware onto both ESP32s. Each board will automatically negotiate its role (transmitter or receiver) at power-on.

## Operating Modes

Before building, set the mode variable in node_final.c to one of the following options:

| Mode | Description |
| --- | --- |
| NORMAL | Normal operation: transmits detection results only |
| DEV_CSI | Streams raw CSI data for development/tuning |
| DEV_VIB | Streams raw vibration data for development/tuning |
| DEV_MIC | Streams raw microphone data for development/tuning |

# 6. Tuning

Sensor thresholds depend on room layout, sensor placement, and floor type. Use the DEV modes with the provided Python scripts to stream and visualize sensor data before finalizing values.

## Microphone Tuning

💡 The microphone is not sensitive to physical setup and may not require tuning.

Steps to tune:

- Build and flash in DEV_MIC mode

- Activate the data_proc_venv Python environment

- Run mic_reader.py (adjust COM port if not on Windows/COM5), make some test sounds, then press Ctrl+C

- Run mic_processor.py (update file paths to your collected data)

A graph such as this one will be produced:

Select a threshold from the graph. Take the square root (convert power to amplitude) and set the macro in sound.c:

// Example: threshold = 3e13, sqrt(3e13) ≈ 5477225

#define MIC_THRESHOLD  5477225

## CSI Tuning

Because it depends on antenna patterns and multipath reflections, CSI is sensitive to room layout and antenna orientation. CSI data is processed using a moving average and rolling variance over two subcarrier groups. The plots below show example raw magnitudes and smoothed data.

Steps to tune:

- Build and flash in DEV_CSI mode

- Run csi_reader.py and csi_processor.py

- A graph such as this one will be produced:

Select two thresholds: one for walking, one for large motion. Both should be above the noise floor. Set the macros in csi_rx.c:

#define CSI_THRESHOLD_LOW   100000   // e.g. 0.1e6 for walking

#define CSI_THRESHOLD_HIGH  650000   // e.g. 0.65e6 for large motion

## Vibration Tuning

The piezo disc is highly sensitive to its physical setup. One possible configuration uses clamps to cantilever the disc with a weight on the end, plus weights to couple it to the floor. Wood floors work best.

💡 Avoid pulldown resistors and biasing on the ADC pin. These attenuate the signal (as shown in image below). An adaptive threshold compensates for the floating pin.

Steps to tune:

- Build and flash in DEV_VIB mode

- Run vib_reader.py and vib_processor.py

- A graph such as this one will be produced:

Adjust thresh_factor until the adaptive threshold (orange line) sits above the noise floor, then set the macro in vib.c:

#define VIB_THRESH_FACTOR  <your_value>

💡 Collect data at different times and with different physical configurations to find the most robust setup.

# 7. Final Product

With the 3D-printed enclosure, the assembled device looks like this:

The program ui.py opens the monitoring dashboard when device is in NORMAL mode:

## Operating the Device

- Ensure both ESP32s are flashed in NORMAL mode.

2.   Power on the transmitter ESP32 first. It will automatically detect no other transmitter and claim that role.

- Power on the receiver ESP32 (with sensors attached). It will find the transmitter and take the receiver role.

- Connect the receiver to a computer via USB and run ui.py to launch the dashboard.

When a fall is detected, the device beeps and the fall indicator on the dashboard lights up red. Presence detection is indicated by the yellow box. A scrolling log shows the events that triggered each detection.

# 8. Resources

## GitHub Repository

github.com/mecargill/thread_proj

## Component Purchases

ESP32 DEVKIT V1 boards

INMP441 Microphone Board

MAX98357 Amplifier Board

Speaker (1W, 8 Ohm)

Piezoelectric Disc

## Espressif / ESP-IDF Documentation

ESP-IDF Getting Started Guide

ESP-IDF GitHub Repository

ESP-IDF API Reference

DOIT ESP32 DEVKIT V1 Pinout

## CSI Sensing Background

How CSI Sensing Works (Digital Intelligence)

CSI Sensing Encyclopedia (Huawei)